2. Hardware resistor on ICHG pin

The BQ25690 also lets you establish the default charge current using a resistor connected to ICHG. TI provides discrete settings:

ICHG resistor	Default charge current

4.64 kΩ	100 mA

6.04 kΩ	500 mA

8.25 kΩ	1.0 A      10k+47k

10.5 kΩ	1.5 A      10k+500R

13.7 kΩ	2.0 A

16.9 kΩ	2.5 A

27.4 kΩ	3.3 A

So if you're building your 1S Li-Po + USB-C PD design and want 2 A charging without relying on firmware, use:

ICHG → 13.7 kΩ → GND

The resistor is referenced to the default 10 mΩ battery sense resistor. If you use a 5 mΩ sense resistor, TI says to set the RBAT_SNS bit so the programmed current remains properly scaled.

For your design

If you're using the BQ25690 with 1S Li-Po, 2 A charging, and 5 V/3 A reverse/boost output, I'd configure it roughly as:

CELL = 4.64 = 4.7k+330k = 1s

VCHG = 13.7k = 15k+150k = 4.2V

ICHG = 8.25k = 10k+47k = 1A