The TI BQ25895 has an input voltage range of 3.9 V to 14 V. TI specifies 22 V as the absolute maximum input voltage.

So, for your USB-C setup:

USB-C PD charger → STUSB4500 → BQ25895 VBUS

For example, if the STUSB4500 negotiates 9 V, the BQ25895 can accept that 9 V input and use it to charge the single-cell Li-ion/Li-polymer battery.

Parameter	BQ25895

Normal input range	3.9–14 V

Absolute maximum	22 V

Recommended high-voltage input	9–12 V

Battery charge voltage	3.84–4.6 V

The BQ25895 is a buck charger, so it can take something like 9 V or 12 V in and step it down to the appropriate battery charging voltage.

If you're designing the STUSB4500 + BQ25895 + battery circuit, I can also explain exactly what voltage should come out of the STUSB4500 and where it connects to the BQ25895.